Appliqued Beach Tote With Zippered Pocket (Optional)

Beginner Sewing 3

Materials (per student)

Main Fabric:

- 2 pieces of 18 inches x 19 inches (body front and back)

- (If adding zipper  - 1 piece of 18” x 19” and 1 piece of 18 inches x 14 inches (body back bottom)+ 1 piece of 6 inches x 18 inches (body back top)

Lining:

- 2 pieces of 18 inches x 19 inches (body front and back)

Other:

- Webbing for straps

- One 7” zipper (if adding zipper):

Class 1

- Use paper and pencils/markers to design bag (show students ideas, students can look through fabrics to plan their design)

- Start applique if ready

Students should be aware that there is a box bottom and the lower 3 inches will be the base of the bag. There is also ½” seam allowance at the top and sides of the bag that won’t show.

Classes 2 and 3– Work on Applique Design

Use fabrics supplied and also FREE SEW box to make applique design on front of bag.

- Think of the background first and add strips of fabric for sand, ocean and sky (if using the beach theme). Either:

  - Turn under the edges and straight stitch

  - Zigzag the raw edges

- Next add the main design, in this case the beach huts

- Fill in the details, for examples the boats, birds, kite, sun and clouds

Class 4 - Decorate Back of Bag

Only advanced students should add zipper (see separate instructions)

Ideas -

Patch Pocket

Felt Lettering

Butterflies cut out of a patterned fabric and appliqued in place using a tight zigzag stitch

Class 5 - Sew Bag Front To Back & Add Box Bottom

(Optional - the box bottom can be skipped if student is behind)

- With RST pin bag front to bag back.

- Straight stitch around 3 edges leaving the top edge unstitched.

Box Corners -

  - Measure 2 ½” from corner and draw a line.

  - Straight stitch along the marked line.

  - Trim the extra fabric off.

  - Repeat this process for other bottom corner.

Class 6 - Lining

- With RST pin lining front to lining back.

- Straight stitch around 3 edges leaving the top edge unstitched. LEAVE A 3” OPENING AT BOTTOM EDGE FOR TURNING

Box Corners - do the same as outer bag

Class 7 – Stitch Lining To Main Bag

- Turn main bag inside out and insert lining so that Right Sides are Together (RST).

- Match top edges of main bag and lining.

- Pin and stitch

- Turn to right side, pulling main bag through opening left in lining

- Iron

- On right side of bag topstitch around top edge. Stitch a second row of top stitching

Class 8 - Straps

Pin and stitch straps in place.

The Beach Tote Bag is complete, congratulations!